Controlling a Car with Hand Motion

Embedded Systems Graduation Project — Technical Documentation

Prepared by: Zyad Hussien Mohamed

Microcontroller Platform: ATmega32

Toolchain: Atmel Studio — Simulation: Proteus

# 1. Project Overview

This project implements a two-module embedded system that allows a remote-controlled car to be driven using natural hand gestures. The system is composed of two independent ATmega32-based modules that communicate wirelessly over Bluetooth (HC-05): a Hand Module worn by the user, and a Car Module mounted on the vehicle.

The Hand Module continuously reads the orientation of the user's hand using an MPU6050 accelerometer/gyroscope module over I2C, converts the tilt readings into a driving command, and transmits that command wirelessly. The Car Module receives the command and drives two DC motors (via an L293D motor driver) accordingly, while displaying the current state on a character LCD.

## 1.1 Key Objectives

- Translate intuitive hand-tilt gestures into car movement commands (Forward, Backward, Left, Right, Stop).

- Establish a reliable wireless link between two independent microcontrollers using HC-05 Bluetooth modules in transparent (transceiver) mode.

- Apply a strict layered driver architecture (LIB / MCAL / HAL / APP) for maintainability and code reuse.

- Build every peripheral driver from the register level upward, without relying on vendor HAL libraries.

## 1.2 Project Scope

The system is split into two physically separate modules, each running its own firmware image on a dedicated ATmega32:

- Hand Module — worn on the hand; reads orientation and transmits driving commands.

- Car Module — mounted on the vehicle chassis; receives commands and actuates the motors.

# 2. System Architecture

## 2.1 Car Module Block Diagram

The Car Module centers on the ATmega32, interfacing with the following peripherals:

| Peripheral | Interface | Purpose |
| --- | --- | --- |
| HC-05 Bluetooth Module | UART (PD0/PD1) | Receives driving commands wirelessly from the Hand Module |
| 16x2 Character LCD | DIO (4-bit mode) | Displays the currently executing command |
| L293D Motor Driver | DIO (direction) + PWM (speed) | Drives two DC motors |
| DC Motors x2 | Via L293D | Vehicle propulsion and steering |
| LED / Buzzer | DIO | Visual/audible feedback (headlights, horn) |

## 2.2 Hand Module Block Diagram

The Hand Module centers on a second, independent ATmega32:

| Peripheral | Interface | Purpose |
| --- | --- | --- |
| HC-05 Bluetooth Module | UART (PD0/PD1) | Transmits driving commands wirelessly to the Car Module |
| MPU6050 (Accelerometer) | I2C / TWI (PC0/PC1) | Reads hand tilt/orientation for gesture detection |

## 2.3 Wireless Communication Concept

Each HC-05 module is wired locally to its own microcontroller's UART pins (TX/RX). No physical wire connects the two HC-05 modules — the wireless link exists entirely inside the Bluetooth radios themselves. From the firmware's perspective, each HC-05 behaves as a transparent UART bridge: any byte sent via UART_voidSendData() on one module is received via UART_u8ReceiveData() on the other, with no additional protocol-handling code required.

| Module | UART Role | Bluetooth Role |
| --- | --- | --- |
| Hand Module | Transmitter only (Send) | Initiates pairing (configurable Master/Slave) |
| Car Module | Receiver only (Receive) | Configurable Master/Slave |

# 3. Layered Software Architecture

All firmware in this project follows a strict four-layer architecture, ensuring that hardware-specific details never leak into application logic, and that each driver can be reused independently across modules or future projects.

| Layer | Responsibility | Examples |
| --- | --- | --- |
| LIB | Platform-wide types and utilities shared by every layer above it. | STD_TYPES.h, BIT_MATH.h, Platform_CFG.h |
| MCAL | Microcontroller Abstraction Layer — direct register-level control of on-chip peripherals. | DIO, UART, TIM0, TIM2, ADC, TWI |
| HAL | Hardware Abstraction Layer — drivers for external components built on top of MCAL. | LCD, DCMOTOR, LED, BUZZER, MPU6050 |
| APP | Application logic — decision-making that ties HAL/MCAL drivers together into project behavior. | Command parser, gesture-to-command mapping |

## 3.1 File Naming Convention

Each driver is split into four files following a consistent pattern, which proved essential for avoiding macro name collisions across drivers (a recurring issue resolved during development, e.g. duplicate TIMSK/TIFR definitions across TIM0 and TIM2):

- <Driver>_REG.h — register addresses and bit positions (hardware facts, never change).

- <Driver>_CFG.h — configurable values (baud rate, prescaler, thresholds).

- <Driver>_interface.h — public function prototypes, fully Doxygen-documented.

- <Driver>_prog.c — implementation.

# 4. Driver Inventory

## 4.1 MCAL Layer

| Driver | Function | Key Design Notes |
| --- | --- | --- |
| DIO | Digital I/O — pin direction and read/write control. | Foundation for all other drivers; simulated and tested first. |
| UART | Asynchronous serial communication for Bluetooth link. | 9600 baud, 8N1; blocking polling implementation via UDRE/RXC flags. |
| TIM0 | 8-bit timer used for blocking millisecond delays (CTC mode). | Prescaler = 64 → 1ms tick via OCR0 = 249 at 16MHz. |
| TIM2 | 8-bit timer used to generate PWM for motor speed control (Fast PWM, non-inverting). | Shared PWM signal (OC2) drives both motors' Enable pins simultaneously. |
| ADC | 10-bit analog-to-digital converter (used in the flex-sensor design phase). | Left-adjusted result (ADLAR=1); only ADCH read for 8-bit resolution. |
| TWI / I2C | Two-wire serial interface, Master mode, used to communicate with the MPU6050. | Verified against a 24C02 EEPROM to confirm correctness of Start/Stop/ACK/NACK sequencing. |

## 4.2 HAL Layer

| Driver | Function | Key Design Notes |
| --- | --- | --- |
| LCD | 16x2 character LCD control in 4-bit mode. | Custom half-byte send + enable-pulse helpers; supports strings, numbers, and custom CGRAM characters. |
| DCMOTOR | Direction and speed control for two DC motors via L293D. | Each direction function explicitly sets all four IN pins to avoid state-dependent bugs. |
| LED / BUZZER | Simple digital output drivers for indicator LED and horn buzzer. | Buzzer driven through an NPN transistor switch due to current requirements exceeding safe MCU pin limits. |
| MPU6050 | Reads raw accelerometer values (X, Y, Z) over I2C for gesture sensing. | Uses ±2g full-scale range; raw axis values used directly as tilt indicators (no trigonometric conversion, to avoid floating-point overhead). |

## 4.3 APP Layer

| Module | Function |
| --- | --- |
| Car Command Parser | Blocking UART receive loop; switch-based dispatch to DCMOTOR and LCD based on the received command byte. |
| Hand Gesture Logic | Reads MPU6050 X/Z axes each loop iteration, applies a threshold comparison to derive a single-character driving command, and transmits it via UART. |

# 5. Communication Protocol

A minimal single-byte ASCII command protocol was defined to keep both firmware images simple to implement and simple to test independently (each module can be exercised with a Virtual Terminal over UART before wireless integration).

| Byte | Command | Car Module Action |
| --- | --- | --- |
| 'F' | Forward | Both motors drive forward at fixed speed |
| 'B' | Backward | Both motors drive in reverse at fixed speed |
| 'L' | Left | Left motor stops, right motor drives forward (pivot turn) |
| 'R' | Right | Right motor stops, left motor drives forward (pivot turn) |
| 'S' | Stop | Both motors stop (free-running, non-braking) |

Design note: motor speed is currently fixed in the Car Module rather than transmitted from the Hand Module. A proportional-speed extension (where tilt magnitude maps to PWM duty cycle) was considered and is documented as future work in Section 7.

# 6. Testing Notes and Known Issues

## 6.1 Verified Working

- DIO, UART, TIM0 (delay), TIM2 (PWM), LCD — confirmed functional in Proteus simulation.

- TWI/I2C driver — independently verified via a full write/read cycle against a 24C02 EEPROM (all status codes matched expected values: START=0x08, SLA+W ACK=0x18, Data ACK=0x28, SLA+R ACK=0x40, and the written byte was read back correctly).

- DCMOTOR direction logic — Forward and Reverse confirmed working; Left/Right turning fixed after correcting a bug where only 2 of 4 direction pins were being set per call.

## 6.2 Open Items

| Issue | Status | Notes |
| --- | --- | --- |
| MPU6050 Proteus simulation returns 0xFFFF (no ACK) | Isolated, not yet resolved | TWI driver independently verified correct via EEPROM test; issue is confined to the MPU6050 simulation model itself, not the driver code. |
| Gesture detection threshold value | Provisional | Currently set to a theoretical estimate (±2500 raw LSB); requires calibration against real hardware or a working simulation model. |
| Tilt-axis convention (which axis maps to which direction) | Assumed | Forward/backward assumed mapped to the Z-axis and left/right to the X-axis; must be confirmed empirically once sensor orientation on the glove/mount is finalized. |
| Reverse motor direction — intermittent non-response in earlier testing | Resolved | Traced to a pin mapping mismatch between DCMOTOR_CFG.h and the Proteus schematic (IN3/IN4 wired to PC6/PC7 instead of PC2/PC3); corrected by aligning the configuration file to the physical wiring. |

# 7. Future Work

- Calibrate MPU6050 tilt thresholds against real hardware readings, replacing the provisional estimate.

- Extend the communication protocol to a two-byte format (command + speed) to enable proportional motor speed control based on tilt magnitude.

- Integrate the buzzer as an automatic reverse-warning horn, triggered directly by the 'B' command.

- Add a communication timeout safeguard on the Car Module so the vehicle automatically stops if no command is received within a defined interval, protecting against Bluetooth link loss.

- Migrate the MPU6050 simulation test to alternate Proteus library sources or real hardware to resolve the outstanding ACK issue.

# 8. Bill of Materials

| Component | Quantity | Used In |
| --- | --- | --- |
| ATmega32 Microcontroller | 2 | Car Module + Hand Module |
| HC-05 Bluetooth Module | 2 | Car Module + Hand Module |
| MPU6050 (GY-521 breakout) | 1 | Hand Module |
| L293D Motor Driver IC | 1 | Car Module |
| DC Motors | 2 | Car Module |
| 16x2 Character LCD (HD44780-compatible) | 2 | Car Module + Hand Module (debug) |
| 2N2222 NPN Transistor | 1 | Buzzer driver stage |
| Buzzer | 1 | Car Module (horn) |
| LEDs | 2+ | Car Module (headlight indicators) |
| Pull-up Resistors (10kΩ) | 2 | I2C bus (SDA, SCL) |
| Chassis, wheels, caster wheel, wiring | As needed | Car Module |